# Mobile Robotics — Line Following, PID Control & Odometry Logging

## Full Code Documentation

Project: Autonomous Line Following + PID + Odometry
Platform: LEGO Mindstorms NXT
Programming Language: NXC (Robot) + Python (Analysis)

Authors:

- אלן חבש

- לין אלואוי

- אחמד סוב לבן

# 0. Project Overview

This repository contains the robot software implementation of a differential-drive LEGO NXT robot capable of:

• Window-based line following (non-PID controller)
• PID-based line following
• Encoder-based odometry
• Data logging to NXT flash memory
• Offline trajectory reconstruction using Python

The goal of this documentation is to explain the software architecture, code structure, and implementation decisions, focusing primarily on reproducibility and understanding of the codebase rather than academic report content.

The repository is structured as a robotics software project, where each module represents a functional layer of the system:

• Sensor processing
• Motion control
• Controller logic
• Logging and odometry
• Offline analysis

# 1. Hardware Assumptions Used by the Code

The code is written for a differential-drive LEGO NXT robot with the following configuration:

## 1.1 Mechanical Structure

• Two independent driving motors (left/right)
• Rear support caster wheel
• Downward-facing NXT Light Sensor
• Forward-facing Ultrasonic Sensor

## 1.2 Physical Parameters Used by Odometry

Wheel Diameter: 56 mm
Wheel Radius: 28 mm
Wheel Circumference ≈ 176 mm

These values are used to convert encoder degrees into traveled distance.

Distance Conversion Formula used in code:

distance_mm = (encoderDegrees / 360) × wheelCircumference

# 2. Software Architecture

The system is divided into logical layers to keep the code modular and reusable.

## 2.1 Sensor Layer

Responsible for reading raw sensor values:

• Light sensor — line detection
• Ultrasonic sensor — obstacle detection
• Motor encoders — odometry input

Typical operations:

Read light value → determine deviation from target
Read ultrasonic distance → trigger stop condition
Read encoder rotations → compute displacement

## 2.2 Control Layer

Two alternative controllers are implemented:

### Non-PID Window Controller

Uses calibrated light ranges (“windows”) where each window applies a predefined steering response.

Advantages:
• Simple integer math
• Fast execution
• Predictable behavior

### PID Controller

Implements a discrete PID loop:

error = targetLight − currentLight
integral += error
derivative = error − lastError

correction = P + I + D

Motor mapping:

leftPower = baseSpeed + correction
rightPower = baseSpeed − correction

PID gains are scaled by ×100 to avoid floating-point calculations.

## 2.3 Motion Layer

Transforms controller output into motor commands.

Responsibilities:

• Clamp motor power to safe range [-100,100]
• Apply left/right power updates
• Maintain stable update rate using loop timing

## 2.4 Odometry & Logging Layer

This layer runs alongside the controller and records robot motion.

Main components:

• Encoder sampling
• Distance estimation
• Timestamp tracking
• File writing to NXT flash

Logging occurs periodically rather than every loop to reduce CPU load and memory usage.

Example CSV output:

time_ms,leftDeg,rightDeg,light,us_cm

Optional extended format:

time_ms,x_mm,y_mm,theta_deg,light,leftDeg,rightDeg

# 3. Code Modules

## 3.1 line_follow_no_pid.nxc

Implements a window-based controller.

Logic:

- Read light value

- Determine which calibrated range the value belongs to

- Apply predefined turn strength

Example behavior:

White region → turn left strongly
Black region → turn right strongly
Center window → move straight

Key Parameters:

BASE_SPEED
Window thresholds
Correction strengths
Loop delay
STOP_DISTANCE_CM

## 3.2 line_follow_pid.nxc

Implements scaled integer PID control.

PID Calculation:

P = (error × KP_TIMES_100) / 100
I = (integral × KI_TIMES_100) / 100
D = (derivative × KD_TIMES_100) / 100

correction = P + I + D

Anti-windup limits prevent excessive integral growth.

Advantages over window controller:

• Continuous steering correction
• Smoother curves
• Higher achievable speed

## 3.3 odometry_logger.nxc

Adds trajectory recording capability.

Initialization:

Reset motor encoders
Record start time
Open log file

Periodic Logging:

Every fixed interval:

Read encoder counts
Convert to distance
Write CSV line

Example logged data:

time_ms,leftDeg,rightDeg,light,distance_cm

Design Considerations:

• Logging interval chosen to balance file size vs trajectory resolution
• File operations kept lightweight to avoid slowing control loop

# 4. Parameter Configuration

## 4.1 Light Sensor Calibration

Typical ranges:

Black ≈ 35
Gray/edge ≈ 48–52
White ≈ 65

Window controller uses threshold zones.
PID controller uses TARGET_LIGHT around the edge region.

## 4.2 Speed vs Stability Tradeoff

Higher BASE_SPEED results in:

• Faster lap times
• Increased oscillations
• Greater sensitivity to tuning

PID controller allows higher speeds due to derivative damping.

# 5. Odometry Model

The robot is modeled as a differential-drive system.

Distance estimation:

LeftDistance = degreesToCm(leftEncoder)
RightDistance = degreesToCm(rightEncoder)

Average forward displacement:

deltaS = (LeftDistance + RightDistance) / 2

If pose tracking is enabled:

theta += (RightDistance − LeftDistance) / wheelBase

Position update:

x += deltaS × cos(theta)
y += deltaS × sin(theta)

# 6. Offline Analysis (Python)

Python scripts load CSV logs and reconstruct trajectories.

Main tasks:

• Parse CSV
• Convert timestamps
• Plot XY trajectory
• Compare runs

Typical usage:

python plot_trajectory.py

Outputs:

• Path visualization
• PID vs Non-PID comparison plots

# 7. Performance Observations

Window Controller:

• Simple and robust
• Fast execution
• Slightly oscillatory at high speed

PID Controller:

• Smooth steering
• Faster lap completion
• Requires gain tuning

Logging System:

• Reliable file creation
• Accurate encoder tracking
• Minimal impact on runtime performance

# 8. Known Limitations

• Odometry accumulates drift over long runs
• Lighting changes affect calibration
• NXT memory limits log size
• File writing frequency must be controlled

# 9. Tools Used

• BricxCC + NXC Compiler
• LEGO Mindstorms NXT Hardware
• Python (matplotlib, numpy) for analysis